| Python Code | Description |
| --- | --- |
| import pandas as pd import numpy as np | This line of code imports NumPy and Pandas, which are both open-source python libraries. |
| project_name = "Project #2: Automation of NDOT Concrete Mix Design" client = "Nebraska Department of Transportation (NDOT)" | This line of code sets the variable of our project name and our client to a string that will be used in the future. |
| cubic_yard_ft3 = 27 unit_weight_water | This line of code sets numerical values that we have already determined into variables that correlate to that numbers meaning. It is calculated with feet per cubic yard and pound per feet cubed respectfully. |
| def calculate_water_weight_Q(cement_A, fly_ash_B, silica_fume_C, other_SCM_D, wc_ratio_E): | This line defines a function that will be used to calculate the water weight given the inputs from the user. |
| total_cementitious = cement_A + fly_ash_B + silica_fume_C + other_SCM_D | The line of code shows the actual numerical calculation that will be used to get the total weight of all of our cement |
| return total_cementitious * wc_ratio_E | This line takes the answer to the calculation we just made above, and multiplies it by the water ratio, which is another input from the user. |
| def calculate_volume_R(cement_A, sg_cement_J): return cement_A / (sg_cement_J * unit_weight_water) | This line of code describes a function that calculates the volume of R, which takes sg_cement and multiplies it by the unit water weight, which is the predetermined value that we made in the beginning of the code. |
|  | This process was then repeated five more times using similar code to the one in red. The only thing that was changed, was the different sg factor. Otherwise every other calculation used the unit weight water to get the volume from the function. |
| def calculate_fine_aggregate_Y(percent_fine_G, sg_fine_N, total_volume_X): | This line of code defines another function that takes our fine aggregate variables and uses them for a calculation. |
| return unit_weight_water * (percent_fine_G / 100) * sg_fine_N * total_volume_X | In this line, the function is returning a calculation that multiples the unit weight water by the percent fine aggregate G divided by 100, and then multiplying again by the specific gravity of the fine aggregate, and the total volume of everything to get a ratio. |
|  | The code in green was then replicated two other times, but instead calculated the course aggregate along with the other aggregate. The functions mathematically remained the same, the input values were just switched to the respective aggregate. |
| project_number = int(input("Enter project number: ")) | This line of code simply assigns a variable to as a float to whatever the user is inputting for that value. The code brings up a prompt, that the user can read, so the appropriate information can be inputted. |
|  | The code in purple was then replicated, for each input that the mix design needs to fulfill all the calculations. These included wc ratio, weights for cement and fly ash, fine and course aggregates, etc. |
| Q = calculate_water_weight_Q(cement_A, fly_ash_B, silica_fume_C, other_SCM_D, wc_ratio_E) | This line of code sets the function that we created to calculate the water weight into a new shortened variable. The one letter variable was used here for simplicity because everything is going to be printed out in the end. |
| R = calculate_volume_R(cement_A, sg_cement_J) | This line calls back our function and takes the calculation it did, and assigns to the our variable R which is the volume for the cement and fly ash. |
|  | The code in purple was then repeated, but just in the case of calling back the respected functions that calculated the other volumes. These variables were stores as S, T, U, V, W and X respectfully. |
| Y = calculate_fine_aggregate_Y(percent_fine_G, sg_fine_N, X) | This line of code calls back the function that calculated the fine aggregate and assigns it to Y |
|  | The same code was used again, but this time, just for the other aggregates in our calculations. |
| print(" NDOT Concrete Mix Design – Weight Summary") | This line gives us somewhat of a header that is introduced before all of our other information is displayed below. |
| print(f"Fine Aggregate (Y): {Y:8.0f} lb") | This line of code serves as an example of what one of the outputs would look like. In all of the printed outputs we rounded to the ones place so there are no decimals present. |
| print("End of Mix Design Summary") | Finally, this line of code ends our outputs with a message that shows all of the outputs have been shown. |